Y7 Bridge It — Diagnostic Rubric

Marking guide · 40 marks · 4 parts

| Part | Marks | Focus |
| --- | --- | --- |
| A — Bridge recognition | 10 | Visual literacy: can student identify bridge types / parts / forces from photos? |
| B — Short answer | 10 | Vocabulary fluency: compression, tension, triangulation, design process, stability. |
| C — Reading a drawing | 9 | Numeracy in a technical drawing: scale reading, dimension interpretation, view identification. |
| D — Drawing task | 11 | Procedural skill: isometric cube (Y method) + truss sketch + force labelling. |

Per-Question Mark Allocation

Part A

A1  —  1 mark

Prompt: Look at Bridge A1 above. Circle or write one word: beam · arch · truss · suspension · cable-stayed.

Award full marks if: TRUSS (the diagonal + vertical web between two horizontal chords is a Pratt truss)

A2  —  1 mark

Prompt: Bridge A2 above — what ONE part is the red arrow pointing to? (choose: deck · pier · abutment · truss · cable)

Award full marks if: DECK (top horizontal surface — where the road or walkway sits)

A3  —  2 marks

Prompt: A3 above shows TWO bridges. Which type is on the LEFT, and which type is on the RIGHT?

Award full marks if: LEFT = ARCH bridge  ·  RIGHT = SUSPENSION bridge

Partial (half): identifies the correct concept but with a minor error. Award 1 for a recognisable attempt.

A4  —  2 marks

Prompt: Bridge A4 above shows a Pratt truss with members coloured RED (compression) and BLUE (tension). Name TWO types of member...

Award full marks if: Top chord + verticals (red members). In Pratt trusses: top chord + verticals in compression; diagonals + bottom chord in tension.

Partial (half): identifies the correct concept but with a minor error. Award 1 for a recognisable attempt.

A5  —  1 mark

Prompt: Bridge A5 above is the Sydney Harbour Bridge. Circle ONE word: is this an ARCH, a TRUSS, or a HYBRID of both?

Award full marks if: HYBRID (steel arch + truss bracing between the chords of the arch)

A6  —  1 mark

Prompt: Look at the 5 shapes in A6 above. Which shape is STRONGEST in engineering — which one cannot be pushed out of shape with...

Award full marks if: TRIANGLE

A7  —  1 mark

Prompt: Picture A7 shows the Leaning Tower of Pisa — a tall tower that began to tilt soon after it was built. Which ONE reason b...

Award full marks if: NARROW BASE ON SOFT GROUND (the base was too small and the ground beneath sank unevenly — low stability)

A8  —  1 mark

Prompt: A8 above shows 3 shapes supporting a book. Which shape held the book BEST — A (flat), B (rolled tube), or C (concertina ...

Award full marks if: B — the ROLLED TUBE (the cylinder distributes compression around its walls; flat paper collapses immediately)

Part B

B1  —  2 marks

Prompt: In your own words, what does the word COMPRESSION mean when engineers use it?

Award full marks if: A pushing force that squashes or shortens a material. (Examples acceptable: book sitting on shelf, spine under weight.)

Partial (half): identifies the correct concept but with a minor error. Award 1 for a recognisable attempt.

B2  —  2 marks

Prompt: In your own words, what is TENSION?

Award full marks if: A pulling force that stretches a material. (Examples acceptable: rubber band, suspension cable, guitar string.)

Partial (half): identifies the correct concept but with a minor error. Award 1 for a recognisable attempt.

B3  —  2 marks

Prompt: What is TRIANGULATION and why do engineers use it?

Award full marks if: Using triangles in a structure to make it rigid. Triangles cannot be deformed without breaking a side, so they lock shape. Squares can collapse, triangles hold firm.

Partial (half): identifies the correct concept but with a minor error. Award 1 for a recognisable attempt.

B4  —  2 marks

Prompt: Name the 4 stages of the design process in order.

Award full marks if: 1) Investigate / Explore  2) Develop ideas / Design  3) Produce / Build  4) Test & Evaluate (order important)

Partial (half): identifies the correct concept but with a minor error. Award 1 for a recognisable attempt.

B5  —  1 mark

Prompt: Name ONE thing that makes a tower or bridge MORE STABLE.

Award full marks if: Accept: wide base · low centre of gravity · triangulation · even weight distribution · stronger materials · bracing

B6  —  1 mark

Prompt: What is the difference between an ELEMENT of design and a PRINCIPLE of design?

Award full marks if: Element = building block (point, line, shape, form, colour, tone, texture). Principle = rule for arranging (balance, unity, proportion, contrast, emphasis, movement, rhythm).

Part C

C1  —  2 marks

Prompt: Below is a scaled drawing of a bridge at 1:5 scale. The drawing is 90 mm long. How long is the REAL bridge?

Award full marks if: 90 mm × 5 = 450 mm (real bridge is 450 mm long)

Partial (half): identifies the correct concept but with a minor error. Award 1 for a recognisable attempt.

C2  —  1 mark

Prompt: Which view is shown on the left in C2: plan view (top down) or elevation view (side-on)?

Award full marks if: Plan view (top-down)

C3  —  2 marks

Prompt: Using the dimension line labels on drawing C3, what is the REAL span of this bridge? (scale 1:10, drawing span = 35 mm)

Award full marks if: 35 mm × 10 = 350 mm

Partial (half): identifies the correct concept but with a minor error. Award 1 for a recognisable attempt.

C4  —  1 mark

Prompt: Drawing C4 labels the top chord with length '60 mm' at 1:5 scale. How long is the real top chord?

Award full marks if: 60 × 5 = 300 mm

C5  —  1 mark

Prompt: In drawing C5, identify ONE part labelled on the plan that is NOT visible in the elevation. (Name the part.)

Award full marks if: Deck cross members (only visible from above) OR the two trusses side-by-side

C6  —  1 mark

Prompt: What does '1:20' scale MEAN? Write in your own words.

Award full marks if: For every 1 unit on the drawing, the real object is 20 units. (Or: drawing is 20× smaller than real.)

C7  —  1 mark

Prompt: What are the 2 MAIN rules of technical lettering? (Circle any 2 of: uppercase / curly / same height / decorative / verti...

Award full marks if: UPPERCASE and SAME HEIGHT (and/or VERTICAL)

Part D

D1  —  4 marks

Prompt: On the isometric dot paper below, draw a CUBE using the Y method. Use a ruler. Mark all 3 axes.

Award full marks if: Expected: Y method start (vertical line + two 30° lines out). All 3 axes clearly visible. Parallel lines parallel. Cube closed (all 6 faces implied). Ruler used (no freehand).

Partial (half): identifies the correct concept but with a minor error. Award 1 for a recognisable attempt.

D2  —  4 marks

Prompt: Sketch a simple truss bridge (side view). Include: top chord, bottom chord, at least 3 verticals, and at least 2 diagona...

Award full marks if: Expected: two parallel chords. ≥3 verticals at roughly equal spacing. ≥2 diagonals connecting chords. No freehand wobbliness — use a ruler. Labels helpful but not required.

Partial (half): identifies the correct concept but with a minor error. Award 1 for a recognisable attempt.

D3  —  3 marks

Prompt: On your truss sketch in D2, mark ONE member that is in COMPRESSION with the letter C. Mark ONE member in TENSION with th...

Award full marks if: Correct compression on top chord OR verticals (for a Pratt). Correct tension on bottom chord OR diagonals (for a Pratt). Two letters clearly placed on DIFFERENT members.

Partial (half): identifies the correct concept but with a minor error. Award 1 for a recognisable attempt.

Tier Placement Thresholds

| Tier | Mark range | % of total | Teaching response |
| --- | --- | --- | --- |
| Extension | 32–40 | 80–100% | Independent research · extended writing target · peer-helper role · advanced truss types |
| Core | 26–31 | 65–79% | Standard brief as written · paired with a Core/Support student for some tasks |
| Core-Support | 20–25 | 50–64% | Scaffolded templates · pre-drawn frames · sentence starters |
| Support | 10–19 | 25–49% | 1:1 conferencing · pre-cut materials · partial folio template · bilingual buddy if EAL/D |
| High Support | 0–9 | <25% | Flag for Learning Support referral · consider Life Skills parallel outcomes · modified task version |

Moderation note (for GTPA / Proficient evidence):

•  Mark 3 focus students FIRST (Amanda / Amaar / Rudransh) and take photos of their papers to embed in GTPA Practice 1 as baseline evidence.

•  Record raw marks + tier in 'Y7-TECH-T2-MARKING-DETAILED-Template.xlsx' — Overview sheet.

•  If borderline between tiers (within 2 marks), default to the lower tier + check-in after L04 — easier to promote than demote.

•  The same diagnostic can be re-used at end of Term 2 (same 40 questions) to measure growth for GTPA Practice 5.