TEACHER COPY — with expected answers

Y7 Bridge It — Diagnostic Test — Teacher Version

Ms Gao · Term 2 2026 · 40 marks · 50 minutes

BEFORE THE LESSON:

•  Prepare 8 bridge photos for Part A — print as A3 or project on slide (1 per question A1–A8).

•  Suggested photos: (A1) Pratt truss rail bridge  (A2) Generic beam bridge with arrow on deck  (A3) Combo photo showing an arch AND a suspension  (A4) Pratt truss with red-circled members  (A5) Sydney Harbour Bridge  (A6) shape choices graphic  (A7) paper tower collapsing  (A8) three paper shapes holding books.

•  Prepare the scaled drawing handout for Part C — a simple 450 mm × 150 mm bridge at 1:5 with dimension lines on the members.

•  Print enough isometric dot paper for Part D.

•  Tier placement bands: 32-40 = Extension · 20-31 = Core · 0-19 = Support.

──────────────────────────────────────────────────────────────────────

Part A — Bridge Recognition (10 marks)

A1  (1 mark)

Look at Bridge A1 above. Circle or write one word: beam · arch · truss · suspension · cable-stayed.

EXPECTED ANSWER: TRUSS (the diagonal + vertical web between two horizontal chords is a Pratt truss)

A2  (1 mark)

Bridge A2 above — what ONE part is the red arrow pointing to? (choose: deck · pier · abutment · truss · cable)

EXPECTED ANSWER: DECK (top horizontal surface — where the road or walkway sits)

A3  (2 marks)

A3 above shows TWO bridges. Which type is on the LEFT, and which type is on the RIGHT?

EXPECTED ANSWER: LEFT = ARCH bridge  ·  RIGHT = SUSPENSION bridge

A4  (2 marks)

Bridge A4 above shows a Pratt truss with members coloured RED (compression) and BLUE (tension). Name TWO types of member that are in COMPRESSION.

EXPECTED ANSWER: Top chord + verticals (red members). In Pratt trusses: top chord + verticals in compression; diagonals + bottom chord in tension.

A5  (1 mark)

Bridge A5 above is the Sydney Harbour Bridge. Circle ONE word: is this an ARCH, a TRUSS, or a HYBRID of both?

EXPECTED ANSWER: HYBRID (steel arch + truss bracing between the chords of the arch)

A6  (1 mark)

Look at the 5 shapes in A6 above. Which shape is STRONGEST in engineering — which one cannot be pushed out of shape without breaking a side?

EXPECTED ANSWER: TRIANGLE

A7  (1 mark)

Picture A7 shows a paper tower falling over. What caused it to fall? Circle ONE: wide base · narrow base · strong walls · gravity alone

EXPECTED ANSWER: NARROW BASE (low stability — high centre of gravity tipped the tower over)

A8  (1 mark)

A8 above shows 3 shapes supporting a book. Which shape held the book BEST — A (flat), B (rolled tube), or C (concertina fold)?

EXPECTED ANSWER: B — the ROLLED TUBE (the cylinder distributes compression around its walls; flat paper collapses immediately)

──────────────────────────────────────────────────────────────────────

Part B — Short Answer (10 marks)

B1  (2 marks)

In your own words, what does the word COMPRESSION mean when engineers use it?

EXPECTED ANSWER: A pushing force that squashes or shortens a material. (Examples acceptable: book sitting on shelf, spine under weight.)

B2  (2 marks)

In your own words, what is TENSION?

EXPECTED ANSWER: A pulling force that stretches a material. (Examples acceptable: rubber band, suspension cable, guitar string.)

B3  (2 marks)

What is TRIANGULATION and why do engineers use it?

EXPECTED ANSWER: Using triangles in a structure to make it rigid. Triangles cannot be deformed without breaking a side, so they lock shape. Squares can collapse, triangles hold firm.

B4  (2 marks)

Name the 4 stages of the design process in order.

EXPECTED ANSWER: 1) Investigate / Explore  2) Develop ideas / Design  3) Produce / Build  4) Test & Evaluate (order important)

B5  (1 mark)

Name ONE thing that makes a tower or bridge MORE STABLE.

EXPECTED ANSWER: Accept: wide base · low centre of gravity · triangulation · even weight distribution · stronger materials · bracing

B6  (1 mark)

What is the difference between an ELEMENT of design and a PRINCIPLE of design?

EXPECTED ANSWER: Element = building block (point, line, shape, form, colour, tone, texture). Principle = rule for arranging (balance, unity, proportion, contrast, emphasis, movement, rhythm).

──────────────────────────────────────────────────────────────────────

Part C — Reading a Scaled Drawing (9 marks)

C1  (2 marks)

Below is a scaled drawing of a bridge at 1:5 scale. The drawing is 90 mm long. How long is the REAL bridge?

EXPECTED ANSWER: 90 mm × 5 = 450 mm (real bridge is 450 mm long)

C2  (1 mark)

Which view is shown on the left in C2: plan view (top down) or elevation view (side-on)?

EXPECTED ANSWER: Plan view (top-down)

C3  (2 marks)

Using the dimension line labels on drawing C3, what is the REAL span of this bridge? (scale 1:10, drawing span = 35 mm)

EXPECTED ANSWER: 35 mm × 10 = 350 mm

C4  (1 mark)

Drawing C4 labels the top chord with length '60 mm' at 1:5 scale. How long is the real top chord?

EXPECTED ANSWER: 60 × 5 = 300 mm

C5  (1 mark)

In drawing C5, identify ONE part labelled on the plan that is NOT visible in the elevation. (Name the part.)

EXPECTED ANSWER: Deck cross members (only visible from above) OR the two trusses side-by-side

C6  (1 mark)

What does '1:20' scale MEAN? Write in your own words.

EXPECTED ANSWER: For every 1 unit on the drawing, the real object is 20 units. (Or: drawing is 20× smaller than real.)

C7  (1 mark)

What are the 2 MAIN rules of technical lettering? (Circle any 2 of: uppercase / curly / same height / decorative / vertical)

EXPECTED ANSWER: UPPERCASE and SAME HEIGHT (and/or VERTICAL)

──────────────────────────────────────────────────────────────────────

Part D — Drawing Task (11 marks)

D1  (4 marks)

On the isometric dot paper below, draw a CUBE using the Y method. Use a ruler. Mark all 3 axes.

EXPECTED ANSWER: Expected: Y method start (vertical line + two 30° lines out). All 3 axes clearly visible. Parallel lines parallel. Cube closed (all 6 faces implied). Ruler used (no freehand).

D2  (4 marks)

Sketch a simple truss bridge (side view). Include: top chord, bottom chord, at least 3 verticals, and at least 2 diagonals. Don't worry about scale — neatness matters.

EXPECTED ANSWER: Expected: two parallel chords. ≥3 verticals at roughly equal spacing. ≥2 diagonals connecting chords. No freehand wobbliness — use a ruler. Labels helpful but not required.

D3  (3 marks)

On your truss sketch in D2, mark ONE member that is in COMPRESSION with a red C. Mark ONE member in TENSION with a blue T. (If no coloured pen, just write the letter.)

EXPECTED ANSWER: Correct compression on top chord OR verticals (for a Pratt). Correct tension on bottom chord OR diagonals (for a Pratt). Two letters clearly placed on DIFFERENT members.

──────────────────────────────────────────────────────────────────────

Marking & Tier Placement Workflow

Option 1: You mark all 28 diagnostic papers in one sitting (~30-45 min).

Option 2: Students self-mark in L02 using the rubric doc — I spot-check 5 at random.

Record each student's raw mark in Y7-TECH-T2-MARKING-DETAILED-Template.xlsx (Overview sheet).

Tier placement:

A — Extension          32–40 marks  Ready for independent research + extended writing

B — Core               26–31       On track for Core expectations — standard brief

C — Core-Support       20–25       Needs scaffolded templates + peer support

D — Support            10–19       1:1 conferencing + pre-cut materials + sentence starters

E — High Support       0–9         Flag for learning support + Life Skills parallel if appropriate